Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#2)

Half and Full Adder in Modelsim

Student Name: Talha Hassan khan Roll No: 24i-6012 Section: CE-A

Date performed: _____________, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

In this task, I designed the Half Adder and Full Adder using Verilog HDL. I implemented both circuits using Gate Level and Dataflow Level modeling. I also wrote testbenches to simulate the designs and verify their outputs for different input combinations.

Code:

HALF ADDER module half_adder_gate(sum, carry, a, b); input a, b; output sum, carry; xor(sum, a, b); and(carry, a, b); endmodule module half_adder_dataflow(sum, carry, a, b); input a, b; output sum, carry; assign sum = a ^ b; assign carry = a & b; endmodule module testbench(); reg a; reg b; wire sum_gate; wire carry_gate; wire sum_data; wire carry_data; half_adder_gate HA1(sum_gate, carry_gate, a, b); half_adder_dataflow HA2(sum_data, carry_data, a, b); initial begin a=0; b=0; #50; a=0; b=1; #50; a=1; b=0; #50; a=1; b=1; #50; end endmodule FULL ADDER module half_adder(sum, carry, a, b); input a, b; output sum, carry; xor(sum, a, b); and(carry, a, b); with dataflow //assign sum=a^b; //assign carry=a&b; endmodule module full_adder(sum, carry, a, b, cin); input a, b, cin; output sum, carry; wire s1; wire c1; wire c2; half_adder HA1(s1, c1, a, b); half_adder HA2(sum, c2, s1, cin); or(carry, c1, c2); //assign carry=c1¦c2; endmodule module testbench(); reg a; reg b; reg cin; wire sum; wire carry; full_adder FA1(sum, carry, a, b, cin); initial begin a=0; b=0; cin=0; #50; a=0; b=0; cin=1; #50; a=0; b=1; cin=0; #50; a=0; b=1; cin=1; #50; a=1; b=0; cin=0; #50; a=1; b=0; cin=1; #50; a=1; b=1; cin=0; #50; a=1; b=1; cin=1; #50; end endmodule

Output:

Full ADDER Half ADDER Full Adder

Task 2:

Description:

In this task, I designed a 3×8 Decoder using Verilog HDL. The circuit was implemented using both Gate Level and Dataflow Level modeling. I created a testbench to simulate the decoder and checked the output for different input combinations.

Code:

module decoder_3X8(a,b,c,y0,y1,y2,y3,y4,y5,y6,y7); input a,b,c; output y0,y1,y2,y3,y4,y5,y6,y7; wire na; wire nb; wire nc; not(na,a); not(nb,b); not(nc,c); and(y0,na,nb,nc); and(y1,na,nb,c); and(y2,na,b,nc); and(y3,na,b,c); and(y4,a,nb,nc); and(y5,a,nb,nc); and(y6,a,b,nc); and(y7,a,b,c); endmodule module testbench(); reg a; reg b; reg c; wire y0; wire y1; wire y2; wire y3; wire y4; wire y5; wire y6; wire y7; decoder_3X8 uut(a,b,c,y0,y1,y2,y3,y4,y5,y6,y7); initial begin a=0; b=0; c=0; #50; a=0; b=0; c=1; #50; a=0; b=1; c=0; #50; a=0; b=1; c=1; #50; a=1; b=0; c=0; #50; a=1; b=0; c=1; #50; a=1; b=1; c=0; #50; a=1; b=1; c=1; #50; end endmodule WITH DATAFLOW module decoder_3X8(a,b,c,y0,y1,y2,y3,y4,y5,y6,y7); input a,b,c; output y0,y1,y2,y3,y4,y5,y6,y7; wire na; wire nb; wire nc; assign na=~a; assign nb=~b; assign nc=~c; assign y0=na&nb&nc; assign y1=na&nb&c; assign y2=na&b&nc; assign y3=na&b&c; assign y4=a&nb&nc; assign y5=a&nb&c; assign y6=a&b&nc; assign y7=a&b&c; endmodule module testbench(); reg a; reg b; reg c; wire y0; wire y1; wire y2; wire y3; wire y4; wire y5; wire y6; wire y7; decoder_3X8 uut(a,b,c,y0,y1,y2,y3,y4,y5,y6,y7); initial begin a=0; b=0; c=0; #50; a=0; b=0; c=1; #50; a=0; b=1; c=0; #50; a=0; b=1; c=1; #50; a=1; b=0; c=0; #50; a=1; b=0; c=1; #50; a=1; b=1; c=0; #50; a=1; b=1; c=1; #50; end endmodule

Output:

Task 3:

Description:

In this task, I designed an 8×1 Multiplexer using Verilog HDL. I implemented the multiplexer using Gate Level and Dataflow Level modeling. A testbench was written to simulate the circuit and verify that the correct input was selected for different select-line combinations.

Code:

module MUX8x1(i0,i1,i2,i3,i4,i5,i6,i7,s0,s1,s2,y); input i0,i1,i2,i3,i4,i5,i6,i7; input s0,s1,s2; output y; wire ns0,ns1,ns2; wire w0,w1,w2,w3,w4,w5,w6,w7; not(ns0,s0); not(ns1,s1); not(ns2,s2); and(w0,i0,ns2,ns1,ns0); and(w1,i1,ns2,ns1,s0); and(w2,i2,ns2,s1,ns0); and(w3,i3,ns2,s1,s0); and(w4,i4,s2,ns1,ns0); and(w5,i5,s2,ns1,s0); and(w6,i6,s2,s1,ns0); and(w7,i7,s2,s1,s0); or(y,w0,w1,w2,w3,w4,w5,w6,w7); endmodule module testbench(); reg i0,i1,i2,i3,i4,i5,i6,i7; reg s0,s1,s2; wire y; MUX8x1 uut(i0,i1,i2,i3,i4,i5,i6,i7,s0,s1,s2,y); initial begin i0=0; i1=1; i2=0; i3=1; i4=0; i5=1; i6=0; i7=1; s2=0; s1=0; s0=0; #50; s2=0; s1=0; s0=1; #50; s2=0; s1=1; s0=0; #50; s2=0; s1=1; s0=1; #50; s2=1; s1=0; s0=0; #50; s2=1; s1=0; s0=1; #50; s2=1; s1=1; s0=0; #50; s2=1; s1=1; s0=1; #50; end endmodule Dataflow module MUX8x1(i0,i1,i2,i3,i4,i5,i6,i7,s0,s1,s2,y); input i0,i1,i2,i3,i4,i5,i6,i7; input s0,s1,s2; output y; wire ns0,ns1,ns2; wire w0,w1,w2,w3,w4,w5,w6,w7; assign ns0 = ~s0; assign ns1 = ~s1; assign ns2 = ~s2; assign w0 = i0 & ns2 & ns1 & ns0; assign w1 = i1 & ns2 & ns1 & s0; assign w2 = i2 & ns2 & s1 & ns0; assign w3 = i3 & ns2 & s1 & s0; assign w4 = i4 & s2 & ns1 & ns0; assign w5 = i5 & s2 & ns1 & s0; assign w6 = i6 & s2 & s1 & ns0; assign w7 = i7 & s2 & s1 & s0; assign y = w0 ¦ w1 ¦ w2 ¦ w3 ¦ w4 ¦ w5 ¦ w6 ¦ w7; endmodule module testbench(); reg i0,i1,i2,i3,i4,i5,i6,i7; reg s0,s1,s2; wire y; MUX8x1 uut(i0,i1,i2,i3,i4,i5,i6,i7,s0,s1,s2,y); initial begin i0=0; i1=1; i2=0; i3=1; i4=0; i5=1; i6=0; i7=1; s2=0; s1=0; s0=0; #50; s2=0; s1=0; s0=1; #50; s2=0; s1=1; s0=0; #50; s2=0; s1=1; s0=1; #50; s2=1; s1=0; s0=0; #50; s2=1; s1=0; s0=1; #50; s2=1; s1=1; s0=0; #50; s2=1; s1=1; s0=1; #50; end endmodule

Output:

Multiplexer